| P.V.P Siddhartha Institute of Technology(Autonomous) |  |  |  |  |  |  |  |  |  |  |  |  |  | Signature of Invigilator with date: |  | Marks Obtained: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Department of Computer Science and Engineering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Course: B.Tech |  | Year: II |  |  | Semester: I |  |  |  |  | Objective: I |  |  |  |  |  |  |  |
| Regulation:PVP20 |  | Maximum Marks:10Marks |  |  |  |  |  |  |  |  |  | Session: F.N |  |  |  |  |  |
| A.Y:2023-24 |  | Date:27-09-23 |  |  |  |  |  |  | Duration: 20 min |  |  |  |  |  |  |  |  |
| Subject Code:20CS3301 |  |  |  | Subject Name: Fundamentals of Digital Logic Design |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Registered Number: |  |  |  |  |  |  |  |  |  |  | Name: |  |  |  |  |  |  |
| Answer all the Questions. Each Question carries ½ Mark 20×½ M=10M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S.No | Question |  |  |  |  |  |  |  |  |  |  |  |  |  | CO | Level | Answer |
| 1. | The representation of hexadecimal number (65F)16 in decimal is |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) (1646)10 |  | b) (4532)10 |  |  |  |  | c) (1631)10 |  |  |  |  | d) (5312)10 |  |  |  |  |
| 2. | The octal equivalent of the given binary number 1011101011 |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 7353 |  | b)1353 |  |  |  |  | c) 5651 |  |  |  |  | d) 5657 |  |  |  |  |
| 3. | In 16-bit 2’s complement representation, the decimal number -28 is |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 1111 1111 0001 1100 |  | b) 0000 0000 1110 0100 |  |  |  |  | c)1111 1111 1110 0100 |  |  |  |  | d)1000 0000 1110 0100 |  |  |  |  |
| 4. | The 10’s complement of 65347 is |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 45652 |  | b) 45655 |  |  |  |  | C) 34653 |  |  |  |  | d) 45654 |  |  |  |  |
| 5. | Perform addition of BCD numbers: 1000 + 0110 |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 0001 0110 |  | b) 0001 0100 |  |  |  |  | c) 0010 0100 |  |  |  |  | d) 0010 0101 |  |  |  |  |
| 6. | Find the complement of the expression (A’+B )( C+D’) is _________ |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) AB’+C’D |  | b) (A + B’)(C’ + D) |  |  |  |  | c) A’B+CD’ |  |  |  |  | d)(A+B)(C+D) |  |  |  |  |
| 7. | If we add an inverter at the output of AND gate, what function is produced? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) NOR |  |  |  |  |  |  | b) Ex-NOR |  |  |  |  |  |  |  |  |  |
|  | c) NAND |  |  |  |  |  |  | d) Ex-OR |  |  |  |  |  |  |  |  |  |
| 8. | In Boolean algebra,(A.A’)+A=? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a)A |  | b) 0 |  |  |  |  | c)A’ |  |  |  |  | d) 1 |  |  |  |  |
| 9. | Which of the following options correctly represents the consensus law of digital circuits? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) AB+A’C+BC=AB+A’C |  |  |  |  |  | b) A’B+A’C+BC=AB+A’C |  |  |  |  |  |  |  |  |  |  |
|  | c) AB+A’C+BC=AB+A’C’ |  |  |  |  |  | d) A’B+A’C+BC=AB+A’C’ |  |  |  |  |  |  |  |  |  |  |
| 10. | There are _____________ Minterms for 3 variables (a, b, c). |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | - a) 4 |  | - b) 6 |  |  |  |  | - c) 8 |  |  |  |  | - d)16 |  |  |  |  |
| 11. | How many NAND gates are required to implement OR gate |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 1 |  | b) 2 |  |  |  |  | c) 3 |  |  |  |  | d) 4 |  |  |  |  |
| 12. | X+XY =X represent which law? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Commutative |  | b) Absorption |  |  |  |  | c) Distributive |  |  |  |  | d) Idempotent |  |  |  |  |
| 13. | In K-map a “octet” is group of __________1’s |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 4 |  | b) 8 |  |  |  |  | c) 2 |  |  |  |  | d) 16 |  |  |  |  |
| 14. | How many OR gates are required to realize Y = AB’ + CF + D? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | - 4 |  |  |  |  |  |  | - 5 |  |  |  |  |  |  |  |  |  |
|  | - 3 |  |  |  |  |  |  | - 2 |  |  |  |  |  |  |  |  |  |
| 15. | The Logic expression F = AB’C+ ABC +A’BC’ is in |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) standard SOP form |  |  |  |  | b) canonical POS form |  |  |  |  |  |  |  |  |  |  |  |
|  | c) canonical SOP form |  |  |  |  | d) standard POS form |  |  |  |  |  |  |  |  |  |  |  |
| 16. | Simplify F(x, y, z) =∑ (0, 2,3,5,6, 7) ------------------------------ |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
| 17. | The output expression for SUM and CARRY of a half adder are __________ |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | - S= A EX-NOR B C=A’B |  | b) S= A EX-OR B C=AB |  |  |  |  | c) S= A OR B C=AB |  |  |  |  | - S= A and B C=AB’ |  |  |  |  |
| 18. | Which of the following is a type of digital logic circuit? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | - Combinational logic Circuits |  | - Sequential logic Circuits |  |  |  |  | c) Both |  |  |  |  | d) None |  |  |  |  |
| 19. | In combinational circuit the output depends only on _____ inputs |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Future |  | b) Past |  |  |  |  | c)Present and past |  |  |  |  | d) Present |  |  |  |  |
| 20. | Which of the following logic circuits accepts three binary digits on inputs, and produces two binary digits, a sum bit and a carry bit on its outputs? |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Half Adder |  | b) Full Adder |  |  |  |  | c)Serial Adder |  |  |  |  | d) Parallel Adder |  |  |  |  |