| P.V.P Siddhartha Institute of Technology(Autonomous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Signature of Invigilator with date: |  | Marks Obtained: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Department of Computer Science and Engineering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Course: B.Tech |  | Year: II |  |  |  | Semester: I |  |  |  | Objective: II |  |  |  |  |  |  |  |  |  |
| Regulation:PVP20 |  | Maximum Marks:10Marks |  |  |  |  |  |  |  |  |  | Session: F.N |  |  |  |  |  |  |  |
| A.Y:2023-24 |  | Date:29-11-2023 |  |  |  |  |  | Duration: 20 min |  |  |  |  |  |  |  |  |  |  |  |
| Subject Code: 20CS3301 |  |  |  |  | Subject Name: Fundamentals of Digital Logic Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Registered Number: |  |  |  |  |  |  |  |  |  |  | Name: |  |  |  |  |  |  |  |  |
| Answer all the Questions. Each Question carries ½ Mark 20×½ M=10M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S.No | Question |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO | Level | Answer |
| 1. | The difference between Half subtractor and full subtractor is __________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Half subtractor has two inputs while full subtractor has four inputs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b) Half subtractor has one output while full subtractor has two outputs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c) Half subtractor has two inputs while full subtractor has three inputs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d) All of the Above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. | The number of input lines in a common BCD to seven segment decoder is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 8 |  |  |  |  |  |  | b) 2 |  |  |  |  |  |  |  |  |  |  |  |
|  | c) 16 |  |  |  |  |  |  | d) 4 |  |  |  |  |  |  |  |  |  |  |  |
| 3. | A Device which converts 2^n inputs to n outputs is called |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Decoder |  |  | b) Encoder |  |  |  | c) Multiplexer |  |  |  |  |  | d) De Multiplexer |  |  |  |  |  |
| 4. | ## How many 3x8 Decoders are required to construct a 4x16 Decoder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 2 |  | b)3 |  |  |  |  | c) 4 |  |  |  |  | d) 5 |  |  |  |  |  |  |
| 5. | If A and B are the inputs of a half subtractor, the Difference expression is given by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) A AND B |  |  |  |  |  |  | b) A OR B |  |  |  |  |  |  |  |  |  |  |  |
|  | c) A XOR B |  |  |  |  |  |  | d) A X-NOR B |  |  |  |  |  |  |  |  |  |  |  |
| 6. | Multiplexer is also known as |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | - Data distributor |  |  |  |  |  | - Data Selector |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Data separator |  |  |  |  |  | - Data connector |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. | How many data select lines are required for selecting 32 inputs? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 1 |  | b) 2 |  |  |  | c) 3 |  |  |  |  |  |  | d) 5 |  |  |  |  |  |
| 8. | Find the number of 2x1 MUX required to implement 8x1 MUX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a)15 |  |  |  |  |  |  | b) 20 |  |  |  |  |  |  |  |  |  |  |  |
|  | c) 5 |  |  |  |  |  |  | d) 7 |  |  |  |  |  |  |  |  |  |  |  |
| 9. | The Logic Circuit whose outputs at any instance of time depends only on the present input but also on the past outputs are called |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Sequential Circuits |  |  |  |  |  |  |  | b) Latches |  |  |  |  |  |  |  |  |  |  |
|  | c) Combinational Circuits |  |  |  |  |  |  |  | d) Flip Flops |  |  |  |  |  |  |  |  |  |  |
| 10. | In S-R Latch , When S=1 and R=1 output Q becomes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 0 |  |  | b) 1 |  |  |  | c) No Change |  |  |  |  |  |  | d)Un determined |  |  |  |  |

| 11. | When Toggle Condition occurs in T Flip Flop |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - a) T=1 |  |  |  |  |  |  |  | - b) T=0 |  |  |  |  |  |  |
| 12. | The Register is a type of ______ |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Sequential Circuit | b) Combinational Circuit |  |  |  |  |  | c) CPU |  |  | d) Latches |  |  |  |  |
| 13. | Which of the following is true about synchronous counters. |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) The output of one flip-flop will act as clock input for next flip-flop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b) All the Flip-flops have common clock pulse. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c) Both a & b. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d) None of the above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. | A Counter is defined as_______________ |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) A register that goes through a prescribed sequences of states upon the application of an input pulse. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b) The group of Latches for storing n- bit of Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c) The group of Flip Flops suitable for storing one bit of Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d) The group of Flip Flops suitable for storing binary Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. | ______________type of shift register that requires the input in serial and produce the output in serial. |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Parallel in/ Serial Out |  |  |  | b) Serial in / Parallel Out |  |  |  |  |  |  |  |  |  |  |
|  | c) Serial in / Serial Out |  |  |  | d) A Bidirectional Shift Register |  |  |  |  |  |  |  |  |  |  |
| 16. | Johnson Counter is also known as____ |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) mod 2n counter |  |  |  |  |  | b) Twisted Ring counter |  |  |  |  |  |  |  |  |
|  | c) Decade Counter |  |  |  |  |  | d) Both a and b |  |  |  |  |  |  |  |  |
| 17. | Which of the following is representing Full modulus and truncated counters respectively? |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Mod-6 and Mod-5 |  |  |  | b) Mod-8 and Mod-16 |  |  |  |  |  |  |  |  |  |  |
|  | c) Mod-16 and Mod-12 |  |  |  | d) None of the above |  |  |  |  |  |  |  |  |  |  |
| 18. | The Number of T- Flip Flops required to design a mod-6 Asynchronous Counter is ___________ |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 2 |  |  | - 4 |  | c) 3 |  |  |  |  |  | d) 10 |  |  |  |
| 19. | In a 6-bit Johnson counter sequence there are a total of how many states, or bit patterns? |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) 2 |  | - 4 |  | c) 6 |  |  |  |  | d) 12 |  |  |  |  |  |
| 20. | In which of the following counter no. of states equal to no. of flip flops required? |  |  |  |  |  |  |  |  |  |  |  | CO1 | L2 |  |
|  | a) Ring Counter |  |  |  | b) Parallel Counter |  |  |  |  |  |  |  |  |  |  |
|  | c) Ripple Counter |  |  |  | d) Johnson Counter |  |  |  |  |  |  |  |  |  |  |