| P.V.P SIDDHARTHA INSTITUTE OF TECHNOLOGY (AUTONOMOUS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| BRANCH : Computer Science and Engineering |  |  | REGULATION : PVP-20 |  |
| Course: B.Tech | SUBJECT : Fundamentals of Digital Logic Design |  |  |  |
| SubjectCode:20CS3301 |  | Year and Semester: II Year / I Sem |  | Section: I |
| Academic Year:2023-24 (Semester-I) |  |  |  |  |
| ASSIGNMENT-I |  |  |  |  |

| Q.NO | QUESTION | CO | LEVEL |
| --- | --- | --- | --- |
| 1 | - Select The base (or radix) of the number system such that the following equation holds is 312/20=13.1 - 73x (in base-x number system) is equal to 54y(in base-y number system), the possible values of x and y are. | 1 | L3 |
| 2 | Consider the following Boolean expression for F(P,Q,R,S)=PQ+P’QR+P’QR’S. Calculate The minimal sum-of-products form of F is | 2 | L3 |
| 3 | - Simplify and draw circuit for simplified expression: A(BC+ABC)+AC - Reduce A’C’+ABC+AC’ to 3 literals ABC’D+A’BD+ABCD to 2 literals A’B(D’+C’D)+B(A+A’CD) to 1 literal | 2 | L3 |
| 4 | - Simplify the following Boolean function using K-Map F(A,B,C)=A’C+A’C+AB’C+BC F(x,y,z)=xy+x’y’z’+x’yz’ - Reduce the following expression using K-map and implement using NAND gates F(A,B,C,D)= ∑(2,3,6,7,8,10,11,13,14) - Reduce the following expression using K-map and implement using NOR gates F=∏M(2,8,9,10,11,12,14) | 2 | L3 |
| 5 | - Given two input bits A and B, produce three outputs X, Y, and Z so that X is 1 only when only when A < B, Y is 1 only when A > B, and Z is 1 only when A = B. - A bank wants to install an alarm system with 3 movement sensors. To prevent false alarms produced by a single sensor activation, the alarm will be triggered only when at least two sensors activate simultaneously. Analyze the scenario and design combinational circuit. | 3 | L4 |