PVP23

Code No: 23ES1304

PVP Siddhartha Institute OF TECHNOLOGY

(Autonomous)

DIGITAL LOGIC & COMPUTER ORGANIZATION

(Common to CSE, IT)

Duration: 3 Hours 								Max. Marks: 70

Note:   1. This question paper contains two Parts A and B.

2.  Part-A contains 10 short answer questions. Each Question carries 2 Marks.

3.  Part-B contains 5 essay questions with an internal choice from each unit. Each

question carries 10 marks.

4. All parts of Question paper must be answered in one place

PART-A					10 x 2 = 20 Marks

| Q.No. | QUESTION | Blooms Level | CO |
| --- | --- | --- | --- |
| 1 a) | Convert the hexadecimal number 3A7F to its binary equivalent. | L2 | CO1 |
| 1 b) | Obtain the 2’s complement for 1101010101. | L1 | CO1 |
| 1c) | Draw the circuit diagram for Full adder. | L1 | CO4 |
| 1d) | Differentiate Synchronous and Asynchronous counter | L2 | CO4 |
| 1e) | Explain the difference between direct addressing mode and indirect addressing mode. | L2 | CO2 |
| 1 f) | Draw the hardware for signed magnitude addition and subtraction. | L1 | CO1 |
| 1 g) | Define Hit Ratio. | L1 | CO1 |
| 1 h) | Define Address Space and Memory Space. | L1 | CO1 |
| 1 i) | Define polling. | L1 | CO2 |
| 1 j) | Define interrupt Cycle. | L1 | CO2 |

PART –B

5x10=50 Marks

|  |  | Blooms Level | CO | Max. Marks |
| --- | --- | --- | --- | --- |
| UNIT-I |  |  |  |  |
| 2(a) | Perform the BCD Addition for the given Numbers i) 984 and 599 ii) 429 and 476 | L2 | CO1 | 5 M |
| 2(b) | Simplify the Boolean expressions to minimum number of literals i) (A + B)(A + C’ )(B’ + C’ ) ii) AB + (AC)’ + AB’C (AB + C) iii) (A+B)’ (A’+B’)’ | L3 | CO1 | 5 M |
| OR |  |  |  |  |
| 3 | Minimize the following expressions using K-map: a) F(W,X,Y,Z) = Σm(4,5,7,12,14,15) +d(3,8,10) b) F(A,B,C,D) = ∑m(1,3,7,11,15) + d(0,2,5) | L3 | CO3 | 10M |
| UNIT-II |  |  |  |  |
| 4(a) | Obtain the function f(A,B,C,D) = Σ(1,2,3,4,6,7,8,10,12,14,15) and design a i) 8X1 MUX ii) 4X1 MUX | L3 | CO4 | 5 M |
| 4(b) | Compare D flip flop and T flip flop. | L4 | CO4 | 5 M |
| OR |  |  |  |  |
| 5(a) | Develop the characteristic equation for JK flip flop. | L2 | CO4 | 5 M |
| 5(b) | Explain 4 bit binary down synchronous counter. | L2 | CO4 | 5 M |
| UNIT-III |  |  |  |  |
| 6 | Demonstrate the organization of a 64 - word register stack | L2 | CO2 | 10 M |
| OR |  |  |  |  |
| 7 | Explain addition and subtraction with 2’s compliment data | L2 | CO2 | 10 M |
| UNIT-IV |  |  |  |  |
| 8 | Explain different types of ROM chips | L2 | CO1 | 10 M |
| OR |  |  |  |  |
| 9 | Analyze the concept of virtual memory with the help of an example. | L4 | CO1 | 10 M |
| UNIT-V |  |  |  |  |
| 10(a) | Differentiate I/O bus verses Memory bus | L4 | CO2 | 5 M |
| 10(b) | Explain the purpose I/O interfaces between internal storage and external I/O devices. | L2 | CO2 | 5M |
| OR |  |  |  |  |
| 11 | Explain the interrupt cycle execution with the help of neat diagram | L2 | CO2 | 10 M |

Course Coordinators

1. Dr. K. Pavan Kumar(IT Dept)

2. Mr P Anil Kumar (CSE Dept)

3. Ms Y Manasa (CSE Dept)

4. Ms P Dedeepya (CSE Dept)

Prof. & Head, Dept. of CSE    		  		Prof. & Head, Dept. of IT