| P.V.P SIDDHARTHA INSTITUTE OF TECHNOLOGY (AUTONOMOUS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| BRANCH : Computer Science and Engineering |  |  | REGULATION : PVP23 |  |
| Course: B.Tech | SUBJECT : Digital Logic & Computer Organization |  |  |  |
| SubjectCode:23ES1304 |  | Year and Semester: II Year / I Sem |  | Section: I/II/III |
| Academic Year:2024-25 (Semester-I) |  |  |  |  |
| ASSIGNMENT-I |  |  |  |  |

| Q.NO | QUESTION | CO | LEVEL |
| --- | --- | --- | --- |
| 1 | A). Convert the following base conversions using Number System. - (365.24)10 to ( ? )2 - (333.45)8 to (?)2 - (A B 7.D)16 to(?)8 B). Perform (-75) – (- 43) using signed 2’s complement. | 1 | L2 |
| 2 | Perform the BCD Subtraction using 10’s complement for the given Numbers i) 984 -599 ii) 429 - 476 | 1 | L2 |
| 3 | A).Simplify and draw circuit diagram for the given Boolean expression: A(BC+ABC)+AC B). Reduce A’C’+ABC+AC’ to 3 literals ABC’D+A’BD+ABCD to 2 literals | 3 | L3 |
| 4 | Convert the following expression into canonical POS F(P, Q, R) = (P+Q) (Q+R’) (P+R’) | 3 | L3 |
| 5 | A). Simplify the following Boolean function using K-Map F(A,B,C)=A’C+A’C+AB’C+BC B). Reduce the following expression using K-map . F=∏M(2,8,9,10,11,12,14)+d(6,13) | 3 | L3 |
| 6 | Implement the logic function using an 4 X 1 Multiplexer. F (A, B, C, D) = ∑m (1,3,4,11,12,13,14,15) | 4 | L4 |
| 7 | Suppose only one multiplexer and one inverter are allowed to be used to implement any Boolean function of n variables. What is the minimum size of the multiplexer needed and explain why? | 4 | L4 |
| 8 | Design a synchronous counter that counts the sequence 0−1−0−2−0−3 and then repeat. | 4 | L4 |